For the MAC computer, you can refer to the following steps to upgrade the firmware manually using the upgrade tool.

Before we do the following steps, please make sure the Mac and the smart device are connected to the same router.

1. Extract the zip file and then double click start.command to run the upgrade tool on your Mac computer.

2. Click the “Scan” button to scan your smart device.

3. Select the smart device and then click “Upgrade” button to upgrade the firmware for the smart device.

Note:

1. On the upgrade tool, it may show an "Upgrade fail", while the firmware is upgraded successfully actually, please do not worry about it. You can check the firmware version from your Kasa App and see whether the upgrading is succeed.

2. If below error "error: [Errno 10013] An attempt was made to access a socket in a way forbidden by its access permissions” occurs, open the Config.json file in Data folder with notepad, and change the 8080 in first line to other ports like 24521 then save it.

If you have any problem when upgrading firmware using the upgrade tool, please refer to the below tips:

a. Fail to run start.command

Use another MAC computer to have a try

b. Cannot discover smart device

1) Make sure the smart device is online and in the same LAN as PC

2) Only enable one network adapter on the PC, others including virtual network adapter should be disabled.

c. Fail to upgrade, timeout

Disable firewall and anti-virus on the PC.

The reason why we need disable firewall and antivirus is that the TCP port 9999 which the smart device and PC use to communicate may be blocked, and because the upgrade is conducted within LAN, so there is no security risk exist after disable firewall and antivirus on the PC.

e. Can't extract the zip file.

Try the other extract programs

d. Other errors

1) Try online upgrade on the Kasa app

2) Get the screenshot of the error message and contact TP-LINK support.